The Z3817a is the weirdest GPSDO I've heard of.  It was apparently used as a reference in a Motorola CDMA radio system.  It's a 'listen only' device that doesn't control the GPS receiver - it only listens to whatever the GPS receiver happens to send.  That can be a problem if the GPS receiver isn't configured quite right.  Keep that in mind.  You might have to set up a 'Y' connection so that the data from the GPS receiver goes to both the Z3817A and to a normal terminal port so you can configure the GPS while the Z3817A listens to the replies.

Hold on tight, it get a little weird from here on.  The pinouts are as follows:

DB25 connector - TIB - P1

1:  V+ (18-36V)

2:  10 MHz #1 Lead A (xformer coupled)

3:  10 MHz #2 Lead A (xformer coupled)

4:  TIMEREF1 Data

5:  TIMEREF1 1 PPS

6:  TIMEREF2 Data

7:  TIMEREF2 1 PPS

8:  Gnd

9:  TIMEREF1-Toggle - GPS / Other

10 - 13:  No connection

14:  Gnd

15:  10 MHz#1 Lead B (xformer coupled)

16:  10 MHz#2 Lead B (xformer coupled)

17:  No connection

18:  TIMEREF1 Return (gnd)

19:  No connection

20:  TIMEREF2 Return (gnd)

21:  V+ (18-36V)

22:  TIMEREF2-Toggle - GPS / Other

23 - 24:  No connection

25:  Gnd

DB-9 Connector - MMI (J1)

1:  No connection

2:  Rx (in) @ 9600 8N1

3:  Tx (out) @ 9600 8N1

4:  No connection

5:  Gnd

6 - 7:  NC

8:  1 PPS out (TTL)

9:  No connection

Note that Pin 14 is NOT a 1 PPS input or output as has been reported.  All connections except for power, MMI Tx/Rx & 10 MHz outputs are TTL.  The TIMEREF and TIMEREF-Toggle leads are opto-isolated from the Z3817A so you have to ground the return lines.

The Z3817A can take it's input from either of TWO reference sources.  The sources can be either GPS or another source - probably rubidium in the original application.  If the source is GPS, you give it both 1 PPS and data.  If the source is rubidium, you just give it 1 PPS.  If the Z3817A is looking at one source and the source disappears, it will automatically switch to the other source after about 4 minutes.  You tell the Z3817A that it's reference(s) are GPS by pulling pin 9 of the DB-25 connector (TIMEREF-Toggle) low or leaving it open.  If the reference(s) are rubidium, you pull this pin high.

It looks like the GPS receiver should be an Oncore UT+  8 channel model or equivalent.  The Z3817A seems to be able to understand all the replies from that model.

Everything here is the best-guess result of testing & head-scratching between myself and Skip Withrow.  The changover between GPS & rubidium has been confirmed, but not explored.  There could be changes in performance (e.g. different phase-lock parameters like damping & time constant).  The other big unknown is a full list of commands that the Z3817A will accept.  Skip & I have tried various commands from the other Z38xx units with good results, but there could be special commands that we know nothing about.  I'd be interested in any contributions or corrections you can provide.  As you've probably found, there's almost no info available on this unit.

On ebay you might find a monitoring program that has limited support for the Z3817A.  Just search for 'Z3817A'.  The author worked with Skip & I to add it to his program, but he doesn't have it quite right.  I haven't checked lately to see if he's got a newer version.  I used SatStat for my monitoring.